Wi-Fi Training Program

Assignment – Module 6

1. Pillars of Wi-Fi Security

Wi-Fi security is based on four fundamental pillars:

- Confidentiality:
Ensures that transmitted data is encrypted so that unauthorized users cannot read it. Technologies like AES encryption in WPA2/WPA3 protect data privacy.

- Integrity:
Ensures that data is not modified or tampered with during transmission. Mechanisms like Message Integrity Check (MIC) help detect changes.

- Authentication:
Verifies the identity of users and devices before allowing network access. This can be done using passwords (PSK) or enterprise authentication (802.1X).

- Availability:
Ensures that network services are accessible to authorized users without disruption, even under conditions like interference or attacks.

Explanation:
All four pillars work together to provide a secure and reliable wireless communication environment.

2. Difference between Authentication and Encryption

| Feature | Authentication | Encryption |
| --- | --- | --- |
| Purpose | Verifies user/device identity | Protects data confidentiality |
| When Used | Before access is granted | During data transmission |
| Example | Password, 802.1X login | AES, WPA2/WPA3 encryption |

Explanation:
Authentication ensures that only legitimate users connect to the network, while encryption ensures that their data remains secure during communication.

3. Differences between WEP, WPA, WPA2, and WPA3

| Feature | WEP | WPA | WPA2 | WPA3 |
| --- | --- | --- | --- | --- |
| Encryption | RC4 | TKIP | AES | AES + SAE |
| Key Type | Static | Dynamic | Dynamic | Dynamic |
| Security Level | Very Weak | Moderate | Strong | Very Strong |
| Authentication | Basic | Improved | Strong | Advanced |

Explanation:
Each version improves upon the previous one. WPA2 introduced strong AES encryption, while WPA3 adds enhanced protection like SAE (Simultaneous Authentication of Equals) to prevent brute-force attacks.

4. Why WEP is Considered Insecure

- Uses weak RC4 encryption algorithm

- Relies on static keys that do not change

- Vulnerable to packet sniffing and key recovery attacks

- Lacks strong integrity protection

Explanation:
Attackers can capture enough packets and easily derive the encryption key, making WEP unsuitable for modern use.

5. Why WPA2 was Introduced

WPA2 was introduced to overcome the limitations of WEP and WPA.

Improvements:

- Uses strong AES (Advanced Encryption Standard)

- Provides better data integrity using CCMP

- Supports both personal (PSK) and enterprise (802.1X) modes

Explanation:
WPA2 became the industry standard due to its strong encryption and reliability.

6. Role of Pairwise Master Key (PMK)

- PMK is a master key derived from:

  - Passphrase (in WPA2-Personal) or

  - Authentication server (in WPA2-Enterprise)

- It is not directly used for encryption but is used to generate:

  - PTK (Pairwise Transient Key) for unicast traffic

  - GTK (Group Temporal Key) for broadcast traffic

Explanation:
PMK ensures secure key generation without exposing the original password.

7. How 4-Way Handshake Ensures Mutual Authentication

The 4-way handshake verifies that both the client and AP have the same PMK.

Process:

- AP sends ANonce

- Client generates SNonce and computes PTK

- AP verifies PTK and sends GTK

- Client confirms installation

Security Mechanism:

- Both sides independently derive the same PTK

- No key is transmitted over the air

- Verification ensures both are legitimate

Explanation:
This process ensures mutual trust and establishes secure encryption keys.

8. What Happens if Wrong Passphrase is Used

- Client generates incorrect PMK

- PTK derived will not match AP’s PTK

- Handshake fails during verification

- Authentication is rejected

Explanation:
Without matching keys, secure communication cannot be established, so access is denied.

9. Problem Solved by 802.1X

802.1X addresses the issue of weak and shared authentication.

Problems solved:

- Unauthorized access

- Shared password risks

- Lack of user-level control

Solution:

- Uses centralized authentication (RADIUS server)

- Provides per-user authentication

- Generates dynamic session keys

Explanation:
It replaces simple password-based access with a secure, scalable authentication framework.

10. How 802.1X Enhances Security

- Uses EAP (Extensible Authentication Protocol) for strong authentication

- Supports certificate-based authentication

- Generates unique encryption keys per session

- Enables centralized policy enforcement

Explanation:
802.1X significantly improves security by ensuring strong authentication, dynamic key management, and better control over network access.