Wi-Fi Training Program

Assignment – Module 4

1. Significance of MAC Layer and its Position in OSI Model

The MAC (Media Access Control) layer is a sublayer of the Data Link Layer (Layer 2) in the OSI model, sitting above the Physical layer.

Significance:

- Controls access to the shared wireless medium (air)

- Provides MAC addressing for device identification

- Handles frame encapsulation and decapsulation

- Ensures reliable delivery using acknowledgments and retransmissions

- Manages collision avoidance using CSMA/CA

Explanation:
Unlike wired networks, wireless devices cannot detect collisions easily. Therefore, the MAC layer plays a critical role in coordinating transmissions and avoiding conflicts, ensuring efficient communication in a shared environment.

2. Frame Format of 802.11 MAC Header

The 802.11 MAC frame is more complex than Ethernet due to wireless requirements.

Main Fields:

- Frame Control (2 bytes):

  - Contains protocol version, frame type (Management, Control, Data), subtype, and flags (To DS, From DS, Retry, Power Management)

- Duration/ID (2 bytes):

  - Indicates how long the medium will be reserved (used in NAV)

- Address Fields (4 addresses possible):

  - Receiver Address (RA)

  - Transmitter Address (TA)

  - Source Address (SA)

  - Destination Address (DA)

  - BSSID (identifies the network)

- Sequence Control (2 bytes):

  - Handles sequencing and fragmentation

- Frame Body:

  - Carries actual payload (data or management info)

- FCS (4 bytes):

  - Error detection using CRC

Explanation:
Multiple address fields are used because frames may pass through APs, requiring both original and intermediate addressing.

3. MAC Layer Functionalities (Management, Control, Data Planes)

Management Plane:

- Network discovery (scanning)

- Authentication and deauthentication

- Association and reassociation

- Beacon transmission

Control Plane:

- ACK and Block ACK

- RTS/CTS (collision avoidance)

- NAV management

- Power save signaling

Data Plane:

- Data transmission between devices

- QoS handling (priority traffic)

- Frame aggregation (A-MSDU, A-MPDU)

- Retransmissions for reliability

Explanation:
Separating functionalities into planes improves organization and allows efficient handling of different network operations.

4. Scanning Process and Types

Scanning allows a client to discover available Wi-Fi networks.

Types of Scanning:

- Passive Scanning:

  - Client listens for beacon frames sent periodically by APs

  - No transmission from client

  - Lower power consumption but slower

- Active Scanning:

  - Client sends Probe Request frames

  - AP responds with Probe Response

  - Faster but consumes more power

Detailed Process:

- Client scans available channels

- Collects information (SSID, signal strength, security)

- Builds list of available networks

- Selects best AP based on signal and policy

Explanation:
Active scanning is commonly used in modern devices for faster connectivity.

5. Client Association Process

The process of connecting a client to an AP involves:

- Scanning: Discover available APs

- Authentication:

  - Open authentication or secure authentication

- Association:

  - Client sends Association Request

  - AP responds with Association Response

- IP Configuration:

  - DHCP assigns IP address

Explanation:
Association establishes a logical connection and allows the AP to track and manage the client.

6. EAPOL 4-Way Handshake

The EAPOL (Extensible Authentication Protocol over LAN) 4-way handshake is used to establish secure encryption keys in WPA2/WPA3.

Steps:

- Message 1 (AP → Client):

  - Sends ANonce (random number)

- Message 2 (Client → AP):

  - Generates SNonce

  - Creates PTK using PMK, ANonce, SNonce

  - Sends SNonce

- Message 3 (AP → Client):

  - Verifies PTK

  - Sends GTK (group key)

- Message 4 (Client → AP):

  - Confirms installation of keys

Keys:

- PMK: Master key

- PTK: Used for unicast encryption

- GTK: Used for broadcast traffic

Explanation:
The handshake ensures secure key exchange without exposing sensitive information.

7. Power Saving Mechanisms in MAC Layer

Power saving is critical for battery-operated devices.

Mechanisms:

- Legacy Power Save:

  - Client enters sleep mode

  - AP buffers data

  - Client wakes periodically to receive data

- U-APSD (WMM Power Save):

  - Efficient delivery using trigger frames

  - Reduces latency and power usage

- TWT (Target Wake Time):

  - Devices negotiate wake/sleep schedule

  - Improves efficiency in dense networks

Explanation:
Modern mechanisms like TWT significantly improve battery life and network efficiency.

8. Medium Access Control Methodologies

Primary Method: CSMA/CA

Steps:

- Listen to channel

- If idle → transmit

- If busy → wait and apply backoff timer

- Transmit after timer expires

Additional Techniques:

- RTS/CTS: Reduces hidden node problem

- NAV: Virtual carrier sensing

Explanation:
CSMA/CA avoids collisions rather than detecting them, making it suitable for wireless environments.

9. Block ACK Mechanism

Block ACK improves efficiency by acknowledging multiple frames at once.

Process:

- Sender transmits multiple frames

- Receiver sends single Block ACK

Advantages:

- Reduces overhead

- Improves throughput

- Essential for high-speed Wi-Fi

Explanation:
This mechanism is widely used in modern Wi-Fi standards for performance optimization.

10. A-MSDU, A-MPDU and A-MSDU in A-MPDU

A-MSDU (Aggregation at higher layer):

- Combines multiple MSDUs into one frame

- Lower overhead but less error resilience

A-MPDU (Aggregation at PHY/MAC level):

- Combines multiple MPDUs

- Each frame has its own error check

A-MSDU in A-MPDU:

- Nested aggregation

- Maximizes efficiency

Explanation:
Aggregation reduces protocol overhead and increases throughput, especially in high-speed networks.