Wi-Fi Training Program

Assignment – Module 5

1. Key Features of Wi-Fi 6, 6E, and 7 and Differences from Wi-Fi 5

Wi-Fi 5 (802.11ac):

- Operates in 5 GHz band

- Uses OFDM

- Supports MU-MIMO (downlink only)

- Max speed ~6.9 Gbps

Wi-Fi 6 (802.11ax):

- Operates in 2.4 & 5 GHz

- Introduces OFDMA (multi-user efficiency)

- Supports uplink and downlink MU-MIMO

- Improved battery efficiency (TWT)

- Better performance in dense environments

Wi-Fi 6E:

- Extends Wi-Fi 6 into 6 GHz band

- Provides more channels and less interference

Wi-Fi 7 (802.11be):

- Supports 320 MHz channels

- Introduces Multi-Link Operation (MLO)

- Uses 4096-QAM for higher data rates

- Ultra-low latency and high throughput

Explanation:
Each generation improves speed, efficiency, and ability to handle more devices simultaneously.

2. Role of OFDMA in Wi-Fi 6

OFDMA (Orthogonal Frequency Division Multiple Access) divides a channel into smaller subcarriers called Resource Units (RUs).

How it works:

- Multiple users share the same channel simultaneously

- Each user is assigned a portion of the spectrum

Benefits:

- Reduced latency

- Improved efficiency

- Better performance in crowded networks

Explanation:
OFDMA allows simultaneous transmission to multiple devices, unlike traditional OFDM which serves one device at a time.

3. Benefits of Target Wake Time (TWT)

TWT allows devices to schedule when they wake up to send/receive data.

Benefits:

- Reduces power consumption

- Improves battery life (important for IoT)

- Reduces network congestion

- Improves efficiency in dense environments

Explanation:
Devices sleep when not needed and wake at scheduled times, making communication more efficient.

4. Significance of 6 GHz Band in Wi-Fi 6E

The 6 GHz band provides:

- More available spectrum

- More non-overlapping channels

- Reduced interference

- Higher data rates

Advantages:

- No legacy device interference

- Supports wider channels (80/160 MHz)

- Ideal for high-bandwidth applications

Explanation:
The 6 GHz band is cleaner and less congested, enabling better performance.

5. Comparison: Wi-Fi 6 vs Wi-Fi 6E

| Feature | Wi-Fi 6 | Wi-Fi 6E |
| --- | --- | --- |
| Frequency | 2.4 & 5 GHz | 2.4, 5 & 6 GHz |
| Range | Better | Slightly lower (6 GHz) |
| Interference | Moderate | Very low |
| Channels | Limited | More channels |

Explanation:
Wi-Fi 6E enhances Wi-Fi 6 by adding a new frequency band with less congestion.

6. Innovations in Wi-Fi 7 (802.11be)

- 320 MHz channels for higher bandwidth

- 4096-QAM for higher data rates

- Multi-Link Operation (MLO)

- Improved MU-MIMO

- Reduced latency

Explanation:
Wi-Fi 7 focuses on ultra-high speed and low latency for applications like AR/VR and gaming.

7. Multi-Link Operation (MLO)

MLO allows devices to use multiple frequency bands simultaneously.

Benefits:

- Higher throughput

- Lower latency

- Improved reliability

Explanation:
Instead of switching between bands, devices can transmit data over multiple links at the same time.

8. Purpose of 802.11k and 802.11v

- 802.11k:

  - Provides neighbor reports

  - Helps client choose best AP

- 802.11v:

  - Assists in network management

  - Suggests optimal AP for roaming

Explanation:
These standards improve roaming decisions and network efficiency.

9. Fast BSS Transition (802.11r)

802.11r enables fast roaming between APs.

How it works:

- Pre-authentication with target AP

- Reduces re-authentication delay

Benefits:

- Seamless roaming

- Low latency handoff

- Ideal for voice/video applications

10. How 802.11k/v/r Work Together

- 802.11k: Provides AP information

- 802.11v: Guides client decisions

- 802.11r: Enables fast transition

Combined Effect:

- Faster roaming

- Reduced latency

- Improved user experience

Explanation:
Together, these protocols ensure smooth and efficient roaming in enterprise networks.